BAHRIA UNIVERSITY, (Karachi Campus)

Department of Software Engineering

# REPORT

Course Title: Computer Communication & Networks	Course Code: CEL 223 Course Instructor: DR. SYED MUHAMMAD KHALIQ-UR-RAHMAN RAAZI Class: BSE-5(B)

Lab Instructor: MUHAMMAD TAYYAB	Date: December 24, 2025

UNIVERSITY NETWORK SYSTEM

# GROUP MEMBERS

| S.NO | Student Name | Enrollment # |
| --- | --- | --- |
| 1 | HAMZA KHALIQ | 02-131232-059 |
| 2 | SHAMRAIZ TASAWAR | 02-131232-112 |
| 3 | HASSAN ASIF | 02-131232-113 |
| 4 | HASSAN FAISAL | 02-131232-131 |

Signed:	Remarks:	Score:

# Table of Content

- Description of the Project	3

- Problem Statement	3

- Network Design Documentation	3

- Explanation of Network Structure	3

  - Logical Segmentation (VLANs 10 - 60)	3

  - Centralized Server Farm (VLAN 60)	Error! Bookmark not defined.

  - IoT Automated Logic & Security Features	4

- Coding and Configuration Details	4

  - Core Switch: Inter-VLAN & EIGRP	4

  - Extended ACL	5

  - Services: DNS, Email & Web Portal	5

- Testing and Results	5

- Protocol Analysis	6

- Testing and Verification	6

  - Connectivity Testing (Ping Scenarios)	6

  - Email Service Testing (SMTP/POP3)	8

  - IoT Logic and Alert Testing	Error! Bookmark not defined.

  - Security & ACL Testing	8

  - Web Portal & DNS Testing	9

  - Advanced Protocol Verification	9

  - File Transfer Testing (FTP)	10

- Conclusion	11

# Description of the Project

The University Campus Networking System (UCNS) is designed to provide a secure, scalable, and efficient network infrastructure for multiple buildings and departments within a university campus. The system integrates VLAN-based segmentation, centralized server access, IoT-based automation, and essential security measures using Cisco Packet Tracer.

# Problem Statement

The campus currently lacks a unified network infrastructure, leading to:

- Inefficient traffic management between buildings.

- Lack of VLAN segmentation for departments.

- No dynamic IP allocation in some buildings.

- Absence of secure communication with external services like cloud-based email.

- Need for scalable, secure, and maintainable network design..

# Network Design Documentation

The network includes multiple VLANs representing different departments, a centralized server farm, IoT devices, and core/access switches. EIGRP or RIPv2 protocols are implemented for efficient inter-VLAN communication. ACLs and IoT logic ensure secure operation.

# Explanation of Network Structure

## Logical Segmentation (VLANs 10 - 60)

    - VLAN 10: Faculty

    - VLAN 20: Labs

    - VLAN 30: Admissions

    - VLAN 40: Library

    - VLAN 50: Student Services

    - VLAN 60: Centralized Server Farm

    - VLAN segmentation isolates departmental traffic for security and efficiency.

  - Centralized Server Farm (VLAN 60)

The Server Room is the backbone of the university network and hosts multiple critical services:

#### DHCP & DNS Server

  - Automatically assigns IP addresses to all VLANs

  - Resolves internal domain name: portal.gamingzone.com

#### Email Server (SMTP/POP3)

  - Enables internal communication and security alert notifications Centralizing servers enhances availability, manageability, and security.

## IoT Automated Logic & Security Features

    - IoT devices monitor campus systems such as lighting and security.

    - Extended ACLs restrict unauthorized access.

    - Alerts are generated and sent to administrators for real-time monitoring.

# Coding and Configuration Details

#### Core Switch: Inter-VLAN & EIGRP

The Core Switch manages all internal routing and communicates with the Edge Router via EIGRP.

! --- VLAN CONFIGURATION --- vlan 10,20,30,40,50,60

exit

! --- LAYER 3 INTERFACES ---

interface Vlan10

ip address 192.168.10.1 255.255.255.0

interface Vlan20

ip address 192.168.20.1 255.255.255.0

ip helper-address 192.168.60.2 interface Vlan30

ip address 192.168.30.1 255.255.255.0

ip helper-address 192.168.60.2 interface Vlan40

ip address 192.168.40.1 255.255.255.0

ip helper-address 192.168.60.2 interface Vlan50

ip address 192.168.50.1 255.255.255.0

ip helper-address 192.168.60.2 interface Vlan60

ip address 192.168.60.1 255.255.255.0

! --- DYNAMIC ROUTING (EIGRP) ---

ip routing router eigrp 100

network 192.168.10.0 0.0.0.255

network 192.168.20.0 0.0.0.255

network 192.168.30.0 0.0.0.255

network 192.168.40.0 0.0.0.255

network 192.168.50.0 0.0.0.255

network 192.168.60.0 0.0.0.255

network 10.0.0.0 0.0.0.3 no auto-summary

#### IoT Security: Extended ACL

Extended Access Control Lists (ACLs) are applied to:

#### Restrict user VLANs (20, 30, 40)

    - Allow Admin VLAN (10) full access for monitoring and control

This ensures that sensitive IoT infrastructure remains protected from regular users.

ip access-list extended IOT_SERVER_RESTRICTION permit ip 192.168.10.0 0.0.0.255 host 192.168.50.10

deny ip 192.168.20.0 0.0.0.255 host 192.168.50.10

deny ip 192.168.30.0 0.0.0.255 host 192.168.50.10

deny ip 192.168.40.0 0.0.0.255 host 192.168.50.10

permit ip any any

!

interface Vlan50

ip access-group IOT_SERVER_RESTRICTION in

#### Services: DNS, Email & Web Portal

The following services are deployed to support daily operations:

    - DHCP: Automatic IP assignment

    - DNS: Name resolution for internal services

    - SMTP/POP3: Email communication and alerts

    - FTP: Cross-VLAN file sharing

# Testing and Results

Comprehensive testing was performed to validate the design:

- Connectivity Tests: Verified IP communication for all users across floors

- Routing Tests: Confirmed EIGRP neighbor relationships

#### IoT Tests:

  - Smoke sensor successfully triggers siren

  - Trip sensor activates alarm

  - RFID access works only for authorized cards

- Security Tests: ACLs successfully block unauthorized VLAN access to IoT servers All test results confirmed correct and stable operation.

# Protocol Analysis

- EIGRP (Enhanced Interior Gateway Routing Protocol): Provides dynamic routing between the Core Multilayer Switch and the Edge Router using the DUAL algorithm for fast convergence and loop-free paths.

- Inter-VLAN Routing: Implemented on the Cisco 3560 Multilayer Switch to enable controlled communication between different VLANs and floors.

- SMTP / POP3: Supports email communication and automated alert notifications across all VLANs..

# Testing and Verification

### Connectivity Testing (Ping Scenarios)

#### MAIN CAMPUS TO KARACHI CAMPUS:

From building A to Building B:

From admin pc to cloud server

#### Edge Connectivity (EIGRP Check)

Test: From the Core Switch, ping the Edge Router's WAN interface.

## Email Service Testing (SMTP/POP3)

#### Cross-VLAN Mail Flow

Sending mail from one vlan to to another

## Security & ACL Testing

    - Restricted Access Check

## Web Portal & DNS Testing

## Advanced Protocol Verification

## File Transfer Testing (FTP)

    - FTP Upload and Download

# Conclusion

The University Campus Networking System successfully implemented a secure, scalable, and efficient network. VLANs, inter-VLAN routing, DHCP, DNS, Email, Web Portal, IoT automation, and ACL security are fully functional. Future enhancements could include redundancy, advanced monitoring tools, VPN-based remote access, and IPv6 integration for modern network demands.